The Four Central Questions of my Data Center Flexibility Idea:

- Technical feasibility of compute migration — Can workloads physically move? What are the bounded limits on latency, ramp rates, workload types, and opportunity costs?

- Grid physical feasibility — When the source zone is overstressed, can we reliably expect power availability at destination zones? What does the empirical pricing record actually show?

- Administrative and market integration — How does this capability fit into PJM’s capacity construct? Given PJM’s existing 92% ELCC class rating for demand response, what commitment depth can a data center fleet credibly sustain, what are the avoided cost implications, and how does the depth of commitment create the incentive structure?

- Mechanical implementation — What does the actual operational protocol look like for both the grid operator and the data center? Dispatch signals, certification requirements, performance assessment, penalty structures.

Context

In working through my framework, I’ve concluded that the right analytical angle is not a novel ELCC methodology as I originally proposed a few weeks ago. PJM already gives demand response a 92% ELCC class rating, derived from a full Loss of Load Probability (LOLP) simulation with DR dispatched last in the stack, modeled with no energy constraint, and available 24/7. That 92% is well-grounded and defensible, and frankly it would be imprudent to go to war with PJM on that point in the context of our EPIC paper. Therefore, the question our analysis should answer is: given that PJM will accredit DC flexibility at 92%, how deeply can a data center fleet credibly commit as demand response?

The specific answer to this question is extremely difficult to pin down with open-source-only information. I’ll go into detail on my scenario-based numbers in a separate document, but the takeaway is that there is a material increase in commitment depth afforded by implementing spatial shifting. In this document I’m going to outline the policy design that enables and incentivizes data center flexible load to reduce the reliability requirement and to add unforced capacity capability to the market.

For my reference and yours, I thought it might be helpful to walk through, in writing, how PJM’s capacity market works, why the 92% ELCC is the correct starting point, how commitment depth differs across mechanism portfolios, and the sequential policy framework that translates flexibility into grid value.

How PJM’s Capacity Market Actually Works

I want to lay this out clearly because the mechanics matter for understanding where data center flexibility fits in.

The Base Residual Auction

PJM runs a forward capacity auction roughly three years ahead of each delivery year. All resource types (gas, nuclear, solar, storage, demand response) compete on the same $/MW-day basis. PJM constructs a Variable Resource Requirement (VRR) curve that represents how much capacity the system needs, and resources bid to fill it. The auction clears at a single price for the RTO, with possible locational adders for constrained zones called Locational Deliverability Areas (of which ComEd territory is now considered one of these constrained zones).

For the 2027/2028 delivery year, the auction cleared at the FERC-approved price cap ($333.44/MW-day) for the entire PJM footprint. This translates to roughly $121,700/MW-year. The cap has been binding for three consecutive auctions, which tells us in dollar terms what we already know from my research in the fall: the system is supply-constrained.

Quick BRA aside: There is currently significant debate over what to do about these massive capacity market clearing prices from an auction standpoint. From what I’ve read, had there not been a price cap in 27/28 auction it is assumed that ratepayers would have been on the hook for $26 billion dollars of capacity procurement. With that cap that hit, they are still on the hook for $16 billion of capacity procurement. From a market perspective, capping the price of the auction blunts the “build here” signal to developers. However, given what I’ve learned in my research over the past few months I do not believe that there even is supply to respond to that signal here in PJM due to a number of hard constraints. With that said, that sets the table for our DR discussion as it should help alleviate some of this capacity market bottleneck in a cost-effective way.

There is no hard MW cap on DR participation. DR offers compete alongside everything else in the capacity auction. In the 2027/2028 BRA, about 7,299 MW of DR cleared (roughly 5% of total cleared capacity).

If you clear in the BRA, you are paid the clearing price for every day of the delivery year. It is not a per-event payment. It is an annual revenue stream (paid monthly) for being available. A 100 MW DR resource with 92% ELCC can offer 92 MW UCAP. At $333.44/MW-day, that’s approximately $11.2 million per year in capacity revenue regardless of whether PJM dispatches you zero times or fifty times. This is one of the reasons why our analysis of peak stress event frequency and duration is so important.

The obligation is that when PJM does call on you during a Performance Assessment Interval (PAI), you must deliver. If you fail, you face Non-Performance Charges that claw back a fee that is 3-4x the capacity payments at large. These penalties are automatic.

Dispatch Frequency

Historically, PJM has called 1–2 emergency dispatches per year in any given zone. Since 2007, PJM has called about 70 total emergency dispatches across the entire footprint. There have been very few mandatory events since 2014. The last major mandatory event was Winter Storm Elliott in December 2022, which lasted approximately 17 hours total.

This is critical for the expected value calculation. You’re collecting ~$11.2M/year in capacity revenue and your expected cost of actual dispatch is maybe 1–2 events × 4–8 hours each × your mechanism cost per hour. Even in a worst-case year like Elliott, you’re talking about 17 hours of dispatch cost against a full year of payment. The revenue-to-cost ratio is massively favorable for any DC that can deliver via any of the three prongs. However, it is worth noting that I do not believe this revenue moves the needle at all for the data centers. This expected value calculation is more applicable in the context of assumed risk for the data center operators.

This is particularly important to consider in the context of what we are discussing because we are essentially advocating to shift a significant amount of risk on to the data centers. As they respond to that shift, they must make their own cost-benefit assessments on how much flexibility they are willing to offer and that will of course depend on probability of getting called (among other factors).

Location and Dispatch

Another important aspect of DR dispatch is that it is location-sensitive. PJM dispatches based on availability, location, dispatch price, and quantity of load reduction needed, subject to transmission constraints. Emergency procedures can be implemented on a Control Zone-specific basis or a subset of a Control Zone. This means a data center in ComEd would primarily be dispatched for ComEd-zone or RTO-wide emergencies, not for a localized congestion event in New Jersey.

How PJM Actually Derives the 92% ELCC Value

Key components of the ELCC model:

- Dispatch Order: In PJM’s ELCC model, DR is dispatched last, literally position 11 out of 11 in the dispatch stack, after thermals, renewables, all storage durations, and hydro. DR is an emergency resource so the model only calls on it after everything else has been exhausted. This means DR only needs to cover residual stress after the entire conventional fleet has fired.

- Load Adjustment Factor: DR availability is not modeled as a fixed block. Available DR = Nominated MW × (Hourly Simulated Load / 50/50 Simulated Load). When load is 6% above the median peak (e.g., 90/10 conditions), the model assumes DR provides 6% more curtailment than nominated, because the load being curtailed from is higher. This Firm Service Level (FSL) treatment inflates DR’s effective contribution during the highest-stress hours.

- 24/7 Availability Window: The jump from 69% to 92% ELCC rating a couple of years ago was not because PJM found DR performs better; it was a direct result of eliminating the DR performance window, making DR a 24/7 resource. Previously the model assumed DR delivered zero outside a limited window (essentially summer afternoons). Once they removed that constraint and DR could be dispatched during winter stress events too, the ELCC jumped 23 percentage points. This was a modeling assumption change, not a performance improvement.

- No Duration Limitation:  In PJM’s dispatch methodology, DR has no energy constraint. Storage resources get elaborate two-part dispatch with state-of-charge tracking. DR just gets dispatched whenever load exceeds available supply. There is no “DR runs out after X hours” mechanic. The model treats DR as if it can sustain delivery indefinitely for any event duration. This is important because it places the nomination decision squarely on the load participant. The question then becomes how can we optimize a combination of enabling resources at the data center level to incentivize them to responsibly propose meaningful MW in DR. This is a question that I will decompose in my Pillar 1 Document.

Prong-Level Saturation Considerations

Of course, not all flexibility mechanisms face saturation equally:

- BTM Battery: No saturation. Uncorrelated, facility-level. Limitation is duration only.

- Temporal/DVFS: No saturation. Uncorrelated, facility-level. Limitation is depth (~25% of load).

- Spatial Migration at Scale: Real saturation with a physical mechanism. If 8 GW of DC load in PJM all tries to migrate out during the same stress event, destination grids absorb load from everyone simultaneously, transmission paths congest, and destinations that were “unstressed” start becoming stressed precisely because everyone is landing on them. This scale issue is a real constraint that I’ll detail a potential solution to in my Pillar 1 document.

The Sequential Framework

This is the primary policy contribution for the EPIC writeup. I want to be clear that on the surface I’m not proposing anything that isn’t already being talked about. What will be novel about the writeup is that I am reiterating the recommendations listed below and providing empirical evidence and modelled analysis for why this can and should be implemented (see Pillar 1 for empirical specifics).

With that said, in my opinion there are two ways DC flexibility can interface with the capacity construct, and they are not competing alternatives, they are different entry points on the same timeline tied to interconnection status.

Option A: DR into the BRA (Post-Interconnection)

The DC is already interconnected and operating on the grid. It nominates MW into the BRA as DR. Its full nameplate load appears in PJM’s load forecast, which inflates the Reliability Requirement. Its DR offer shows up as supply that helps meet that requirement. It collects capacity payments. This is PJM’s current framework. The flexibility shows up on the supply side of the ledger. The self-licking ice cream cone problem exists but is economically sound. Once cleared as a resource in the BRA, the commitment is binding and backed by Non-Performance Charges.

Option B: Connect-and-Manage / Flexible Interconnection (Pre-Interconnection)

The DC is in the interconnection queue. PJM says “we need 3–5 years and $200M+ in network upgrades before you can energize at 500 MW firm.” The DC says “what if I interconnect now at 500 MW but with a binding commitment to curtail to 229 MW during stress events?” PJM can approve this because the system only needs to accommodate 229 MW of firm load; the upgrade requirements are smaller or eliminated. The DC gets online years faster. The flexibility commitment is baked into the interconnection agreement itself. PJM’s load forecast reflects the effective firm service level (229 MW), not the nameplate (500 MW). The Reliability Requirement is lower. No DR offer. No capacity payment. No self-licking ice cream cone.

This model connect and manage model already exists in ERCOT for generators. Discussions are ongoing in ERCOT to add this as a demand side policy as well. I think this should be actively discussed in concert with the bring your own capacity notion in PJM. ERCOT’s connect-and-manage approach does not require network upgrades for interconnections, instead managing grid congestion through redispatch and curtailment. ERCOT brought 14.2 GW online in the last two years using this framework, more than any other grid operator. A November 2025 proposal at ERCOT explicitly extends this to load, creating “the basis of connect-and-manage of load using the existing system that ERCOT already has for generators and batteries.”

The key insight is that these are sequential, not competing mechanisms. The choice between Option A and Option B is not a policy design choice. It is a function of where you are in the interconnection process.

Phase 1 — Pre-Interconnection (Queue Purgatory): Option B is the only available path. You cannot offer DR because you don’t exist on the grid yet. But flexible interconnection gets you online years faster. The “payment” is the NPV of accelerated operations, which dwarfs any capacity payment by a massive amount.

Phase 2 — Post-Interconnection: Now you have a choice. Stay on flexible interconnection (no capacity payment, but flexibility is already reflected in reduced demand), or convert to firm service and offer DR into the BRA (collect capacity revenue, but your full load now increases the Reliability Requirement). The economically rational choice depends on whether the capacity payment exceeds the value of the demand-side accounting treatment. Given that the interconnection NPV advantage has already been captured in Phase 1, the Phase 2 decision is purely about annual revenue vs. operational simplicity. I do not think there will be significant action in the “Phase 2 Decision”….at least not until firm BTM supply chain constraints relieve or SMRs become a thing.

The Double-Counting Constraint: A given MW of flexibility can only be counted once. If your interconnection agreement already commits 271 MW of flexibility (and PJM’s load forecast reflects 229 MW firm load for a 500 MW facility), you cannot also offer that same 271 MW as DR into the BRA. That would count the same flexibility twice—once as reduced demand, once as added supply. To participate in the BRA, you’d need to convert to full firm service (putting 500 MW back into the load forecast) and then offer DR.

Impact on the Reliability Requirement

PJM’s own 2025 ELCC/RRS study projects a UCAP surplus of only 1,059 MW for the 2027/2028 delivery year, deteriorating to a deficit of 26,734 MW by 2035/2036. These projections assume the load forecast at face value: DC load at full nameplate draw, 24/7.

Under Option A (DR into BRA), this deficit doesn’t improve on the demand side. The full DC load is in the forecast. DC DR simply adds to supply but the requirement stays high.

Under Option B (connect-and-manage), the deficit improves on the demand side. With the DVFS + Spatial portfolio enabling material commitment depth for inference-dominant facilities, a the aggregate fleet of data centers can offer a portion of nameplate, removing a significant amount of non-firm load directly from the Reliability Requirement before any supply-side DR enters the picture. Because each MW of load removed from the planning process avoids building (1 + IRM) MW of installed generation capacity, the total avoided installed capacity is massive and represents an avoidance or delay of gas turbine buildout. This is a fundamentally different accounting outcome with identical physical grid behavior.

Non-Firm Service: Scenario Breakdown (Directional Values Only)

A critical implementation question: if a DC connects under a non-firm agreement (Option B), how does that interact with capacity market participation over time? PJM does not currently have a connect-and-manage framework for load, and ERCOT’s proposal is still being formalized. But we can reason through the scenarios that any implementation would need to address.

Scenario 1: Stay non-firm, no BRA participation. The DC operates at 500 MW nameplate with 271 MW interruptible (54% commitment depth). The load forecast shows 229 MW. No capacity payment. The “payment” was getting online faster. This is the cleanest version with no accounting complications.

Scenario 2: Stay non-firm, bid incremental flexibility into BRA. Say the DC can actually curtail more than the 271 MW committed in the interconnection agreement via BTM buildout improvements either on site or in DC network over time. The interconnection agreement already commits 271 MW (reflected as reduced demand). Any flexibility above that is genuinely incremental. In principle, the DC could offer this incremental amount as DR into the BRA and collect capacity revenue without double-counting. However, the accounting verification—proving the DR offer is genuinely additional to the interconnection commitment—would require new PJM mechanisms.

Scenario 3: Convert to firm, then bid full DR. Once the transmission upgrades are built (either through PJM’s RTEP process or self-funded), the DC renegotiates to full firm service. Now 500 MW is in the load forecast. The Reliability Requirement goes back up. But the DC can offer the full flexibility as DR. The DC has already captured years of accelerated operations from Phase 1. The non-firm agreement persists until this underlying transmission constraint is resolved. This dampens the time constraint we have talked about.

Scenario 4: This is the one that must be prohibited. Convert to firm service without actually needing the upgrades because you’re going to curtail anyway. This would game the system: the DC inflates the Reliability Requirement by 271 MW (converting from 229 to 500 firm) and then “solves” that inflated requirement by bidding 271 MW of DR. Net grid impact: zero. But the DC has collected capacity payments for flexibility it was already providing for free. From an implementation standpoint this is something I am still trying to game out in my head to determine sufficient mitigation.

The strategic reality is that the capacity payment is a rounding error relative to the value of getting online. These hyperscalers are not resource-constrained on their balance sheets but nobody can do anything about time and supply chains. Flexible interconnection buys them the one thing money alone cannot: speed.

Preview of Long-Term Vision:

The sequential framework (Option B → Option A) describes the near-to-medium term. But there is a longer arc that I find fascinating to think about:

If small modular reactors (SMRs) or other behind-the-meter generation becomes viable, the data center’s relationship with the grid inverts entirely. A DC with BTM generation sufficient to cover its own load could theoretically nominate its entire load—and then some—as DR, then sell surplus generation to the grid in real time. At that point the DC is not demand response. It is a dispatchable supply resource with a captive load that it can modulate in either direction: consume less from the grid, or inject surplus into it. That is a fundamentally different resource class than anything PJM currently categorizes at scale.

The economics could work. The capacity payments and energy market revenue from selling surplus power offset the CapEx of the SMR. The DC gets BTM power assurance (which they want anyway for reliability and operational independence) and gets paid for the excess. Build enough of these facilities and connect them via private fiber-optic networks, and you have a virtually connected power grid optimized specifically for data center operations—one that uses the general electricity grid opportunistically depending on conditions.

If you think about it, this trajectory would create a genuine competitive pressure mechanism on utilities. Today, utilities have a captive customer base and a guaranteed rate of return on infrastructure investment. They have little incentive to accelerate interconnection or modernize grid operations. But if hyperscalers can demonstrate a credible path to energy independence via BTM generation, private campus networks, geographic load optimization across facilities, then the utility’s value proposition narrows to “we provide the last-mile wire.” The utility either accelerates interconnection and service quality to keep these large customers on the grid and paying tariffs, or watches them build around the utility entirely. That is a market discipline mechanism that does not exist today, and it emerges directly from the flexibility capability this thesis describes. Pretty crazy to think about.

Additional References

E3 Illinois RA Study: Illinois Power Agency, Illinois Commerce Commission, and Illinois 	Environmental Protection Agency. 2025 Resource Adequacy Study. Prepared by 	Energy and 	Environmental Economics, Inc. (E3). December 15, 2025. 	https://ipa.illinois.gov/content/darn/soi/en/web/ipa/documents/20251215-illinois-	ra-study-2025-	final.pdf.

Emerald AI/ Nature Energy: Colangelo, Philip, et al. "AI Data Centres as Grid-Interactive Assets." Nature 	Energy (2025). https://doi.org/10.1038/s41560-025-01927-1.

ERCOT Connect-and-Manage: Howland, Ethan. "Can ERCOT Show the Way to Faster and Cheaper 	Grid Interconnection?" Utility Dive, November 27, 2023.  	https://www.utilitydive.com/news/connect-and-manage-grid-interconnection-ferc-ercot-	transmission-planning/698949/.

PJM 2027/2028 BRA Report: PJM Interconnection. 2027/2028 Base Residual Auction
	Report. Valley Forge, PA: PJM Interconnection, December 17, 2025. https://www.pjm.com/-	/media/DotCorn/markets-ops/rprn/rpm-auction-info/2027-2028/2027-2028-bra-report.pdf.

PJM BRA Press Release: PJM Interconnection. "PJM Auction Procures 134,479 MW of	Generation 	Resources." News release, December 17, 2025. 	https://insidelines.pjm.corn/pjm-auction-	procures-134479-mw-of-generation-resources/.

PJM ELCC/RRS Study: PJM Interconnection. 2025 PJM Effective Load Carrying Capability and Reserve 	Requirement Study. Valley Forge, PA: PJM Interconnection, 2025.
	https://www.pjm.com/-/media/DotCorn/planning /res-adeq/elcc/2025-pj m-elcc-	rrs.pdf.

PJM Manual 13: PJM Interconnection. PJM Manual 13: Emergency Operations. Rev. 86. Valley Forge, 	PA: PJM Interconnection, 2025

PJM Manual 18: PJM Interconnection. PJM Manual 18: PJM Capacity Market. Rev. 48. Valley Forge, 	PA: PJM Interconnection, 2025.

PJM Manual 20A: PJM Interconnection. PJM Manual 20A: Resource Adequacy Analysis. Rev. 2. Valley 	Forge, PA: PJM Interconnection, 2025.https://www.pjm.com/ 	/media/DotCorn/documents/manuals/m20a.ashx.

Winter Storm Elliott Report: PJM Interconnection. Winter Storm Elliott Event Analysis and 	Recommendation Report. Valley Forge, PA: PJM Interconnection, July 2023. 	https://www.pjm.com/markets-and-operations/winter-storrn-elliott.

PJM Interconnection, 2025 PJM Effective Load Carrying Capability and Reserve Requirement Study (Valley Forge, PA: PJM Interconnection, 2025), https://www.pjm.com/-/media/DotCom/planning/res-adeq/elcc/2025-pjm-elcc-rrs.pdf.

PJM Interconnection, "PJM Auction Procures 134,479 MW of Generation Resources," news release, December 17, 2025, https://insidelines.pjm.com/pjm-auction-procures-134479-mw-of-generation-resources/.

The 2025/2026 BRA cleared at $269.92/MW-day (the effective cap under pre-reform VRR curve parameters); the 2026/2027 BRA cleared at the explicit cap of $329.17/MW-day; the 2027/2028 BRA cleared at $333.44/MW-day. See PJM Interconnection, 2027/2028 Base Residual Auction Report (Valley Forge, PA: PJM Interconnection, December 17, 2025),  https://www.pjm.com/-/media/DotCom/markets-ops/rpm/rpm-auction-info/2027-2028/2027-2028-bra-report.pdf.

The cleared supply times the clearing price totals $16.4 billion. PJM Interconnection, 2027/2028 Base Residual Auction Report, December 17, 2025. For the uncapped counterfactual estimate, see analysis of the 6,623 MW shortfall relative to the reliability requirement in the same report. The $26 billion figure circulates in industry commentary but should be characterized as an estimate; verify against your preferred source or flag as approximation.

PJM Interconnection, "PJM Auction Procures 134,479 MW of Generation Resources," December 17, 2025. The cleared resource mix included 5% demand response, with DR rising from 5,531 MW in the 2026/2027 auction to 7,299 MW due to increased ELCC ratings.

PJM Interconnection, PJM Manual 18: PJM Capacity Market, rev. 48 (Valley Forge, PA: PJM Interconnection, 2025), Section 11: Non-Performance Assessment. The Non-Performance Charge Rate is derived from Net CONE x the number of Performance Assessment Hours, resulting in potential charges that substantially exceed annual capacity revenues.

PJM Interconnection, PJM Manual 13: Emergency Operations, rev. 86 (Valley Forge, PA: PJM Interconnection, 2025); PJM Interconnection, State of the Market Report (Monitoring Analytics, various years). See also Pillar 2 stress analysis, which derives 16 events/year and 122 dispatch hours/year from empirical data.

PJM Interconnection, Winter Storm Elliott Event Analysis and Recommendation Report (Valley Forge, PA: PJM Interconnection, July 2023), https://www.pjm.com/markets-and-operations/winter-storm-elliott. The storm affected the PJM region December 23-25, 2022, with emergency procedures spanning approximately 17 hours.

PJM Interconnection, PJM Manual 13: Emergency Operations, rev. 86 (Valley Forge, PA: PJM Interconnection, 2025), Section 2: Emergency Procedures.

PJM Interconnection, 2025 PJM Effective Load Carrying Capability and Reserve Requirement Study, 2025. See dispatch order assumptions in ELCC methodology documentation.

PJM Interconnection, PJM Manual 20A: Resource Adequacy Analysis, rev. 2 (Valley Forge, PA: PJM Interconnection, 2025), https://www.pjm.com/-/media/DotCom/documents/manuals/m20a.ashx.

PJM Interconnection, 2025 PJM ELCC/RRS Study, 2025: "The increase is a direct result of eliminating the DR performance window (making DR a 24/7 resource)." See also American Public Power Association, "PJM Details Auction Procurement Results," December 17, 2025, https://www.publicpower.org/periodical/article/pjm-details-auction-procurement-results-says-capacity-resources-procured-plus-frr-resources-short.

PJM Interconnection, 2025 PJM ELCCIRRS Study, 2025. Storage resources are modeled with two-part dispatch and state-of-charge tracking; DR is modeled without an energy constraint and dispatched whenever load exceeds available supply.

Ethan Howland, "Can ERCOT Show the Way to Faster and Cheaper Grid Interconnection?," Utility Dive, November 27, 2023, https://www.utilitydive.com/news/connect-and-manage-grid-interconnection-ferc-ercot-transmission-planning/698949/.

Ibid. Data from S&P Global, as reported in Howland (2023). ERCOT interconnected 14.2 GW in 2021-2022, compared to 5.6 GW for PJM over the same period. See also Lawrence Berkeley National Laboratory interconnection analysis confirming ERCOT's continued outperformance through 2024.

PJM Interconnection, 2025 PJM Effective Load Carrying Capability and Reserve Requirement Study, 2025, https://www.pjm.com/-/media/DotCom/planning/res-adeq/elcc/2025-pjm-elcc-rrs.pdf. See forward-year projections in Section: Delivery Years 2028/2029 Through 2035/2036.

PJM does not currently have a formal connect-and-manage framework for load. The scenarios described here are the author's analytical construction based on existing PJM tariff provisions and the ERCOT precedent. See PJM Interconnection, "Large Load Additions: Stakeholder Feedback," presented September 15, 2025; White & Case LLP, "Grid Operators Propose Innovative Measures to Manage Electricity Demand from Data Centers," 2025, https://www.whitecase.com/insight-alert/grid-operators-propose-innovative-measures-manage-electricity-demand-data-centers.

For discussion of behind-the-meter nuclear as a data center power source, see, e.g., Department of Energy, "Pathways to Commercial Liftoff: Advanced Nuclear" (Washington, DC: DOE, 2024).