# Technical Appendix: Cross-Region Stress Decorrelation Analysis

# Technical Appendix: Cross-Region Stress Decorrelation Analysis

This document provides the complete methodology, results, and interpretation of the cross-region stress decorrelation analysis underpinning the cascade model’s destination availability parameter (D1) and the conditional Monte Carlo simulation. It serves as the extended companion to NE paper §3.

## 1. Research Question

Spatial compute migration requires that at least one destination grid region is operating under normal conditions when the source region is stressed. This document establishes the empirical basis for that condition using four years of day-ahead electricity pricing data across five U.S. grid regions.

The central question: during ComEd stress events, is there always somewhere else to send the workload?

## 2. Data

### Price Data

35,060 hours of day-ahead locational marginal pricing (DA LMP) data from January 2022 through December 2025, spanning five U.S. grid regions:

| Region | Source | Resolution | Timezone Conversion |
| --- | --- | --- | --- |
| PJM | PJM Data Miner | Hourly | Native EPT |
| ERCOT | ERCOT market portal | Hourly | UTC → EPT |
| CAISO | CAISO OASIS | Hourly | Pacific → EPT |
| MISO | MISO market portal | Hourly | Native EPT |
| NYISO | NYISO market portal | Hourly | Native EPT |

All prices are aligned to a common Eastern Prevailing Time datetime index. CAISO coverage begins November 2022 (26,541 overlapping hours); all other regions have full 2022–2025 coverage.

WECC bilateral trading hubs (Mid-Columbia, Palo Verde) are excluded from the current analysis because their published price data is daily-resolution, insufficient for hourly stress event detection. These hubs represent approximately 10,100 MW of current data center capacity; their inclusion would increase destination headroom and shift fleet commitment depth upward.

### Data Center Location Data

County-level data center capacity from two sources:

- Baxtel commercial data center database (baxtel.com) — facility-level records of operational, under construction, and planned capacity

- DOE/NREL Accelerating Speed to Power Data Viewer (maps.nrel.gov/speed-to-power) — county-level capacity aggregation from the “Data Center Demand Capacity by County” layer, accessed December 2025

National totals from the combined dataset: 51,058 MW operational, 36,626 MW under construction, 201,570 MW planned. Only operational capacity is used for the stress correlation analysis, as these facilities were drawing load during the 2022–2025 pricing period.

### County-to-Pricing-Zone Mapping

Each county in the dataset is mapped to a pricing zone:

- PJM counties → PJM settlement zone (ComEd, DOM, AEP, BGE, PECO, PSEG) based on utility territory

- ERCOT counties → ERCOT load zone (LZ_NORTH, LZ_SOUTH, LZ_WEST, LZ_HOUSTON) based on county-to-load-zone geographic mapping

- CAISO counties → NP15 or SP15 based on latitude (NP15 covers Northern California including Santa Clara County / Silicon Valley)

- MISO counties → MISO commercial pricing hub based on state. Six hub-keyed zones: Minnesota Hub (MN, WI, ND, SD), Indiana Hub (IN), Illinois Hub (IL Ameren territory), Michigan Hub (MI), Arkansas Hub (AR), Louisiana Hub (LA, MS)

- NYISO counties → NYISO zone based on geographic territory (Zones A, F, G, J, K)

This mapping is stored in county_node_mapping.py and dc_capacity_mapped.csv.

MISO zone structure note: The current analysis uses six hub-keyed zones rather than the eight MISO Local Resource Zones (LRZs) used in earlier versions. This change was made because MISO publishes day-ahead hub LMPs (not LRZ-level LMPs), making hub-keyed zones the correct unit for price-based stress identification. Two zones with operational data center capacity — Zone 2/Wisconsin (~24 MW operational, ~7,600 MW announced) and Zone 5/Missouri (~411 MW operational) — are excluded because MISO does not publish dedicated hub LMPs for these areas. Iowa counties are mapped to Minnesota Hub (the nearest published hub) rather than to Michigan Hub as in earlier versions.

## 3. Methodology

### Stress Hour Identification

Stress hours are defined as the top 50 DA LMP hours per year per zone (approximately the 99.4th percentile of each zone’s own four-year distribution), yielding 200 stress hours per zone over the 2022–2025 period.

Why price-based (not demand-based): Price captures both demand-driven stress (high cooling/heating load) and supply-driven stress (generator outages, gas curtailments, transmission constraints). A high DA LMP results from either cause; both represent grid conditions under which spatial migration would be triggered. A price-only signal captures the union of stress causes without needing to separate them.

Why top 50 per year: The threshold yields ~50 hours per year, calibrated to existing resource adequacy protocols. PJM’s Performance Assessment Hours (PAH) typically flag 30–50 hours per year. Top-50 is in that range while ensuring adequate sample size for cross-zone coincidence calculation.

Threshold sensitivity: The cross-region decorrelation result is robust to threshold specification. Sensitivity analysis using top-25 (~99.7th percentile) and top-100 (~98.9th percentile) thresholds (reported in Extended Data Fig. 1 of the NE paper) confirms that the qualitative finding — cross-interconnection destinations are rarely co-stressed with ComEd — holds across thresholds.

### Destination Pool

The destination pool comprises 17 pricing zones across four grid regions:

| Region | Zones | Operational DC Capacity (MW) |
| --- | --- | --- |
| ERCOT | LZ_NORTH (4,275), LZ_SOUTH (2,866), LZ_WEST (2,480), LZ_HOUSTON (890) | 10,511 |
| CAISO | NP15 (1,311), SP15 (397) | 1,708 |
| MISO | Minnesota (2,988), Indiana (599), Illinois (66), Michigan (126), Arkansas (305), Louisiana (7) | 4,091 |
| NYISO | F (548), J (191), G (86), A (56), K (10) | 891 |
| Total | 17 zones | ~17,200 MW |

Source zone: PJM ComEd (Northern Illinois), approximately 1,647 MW of operational data center capacity. Cook County alone has 1,069 MW; DuPage, Will, Kane, and surrounding collar counties add the rest.

Intra-PJM control group (5 zones, not used for migration but essential for proving the correlation baseline): DOM/Dominion Virginia (9,431 MW), AEP/Ohio (3,402 MW), PECO/Philadelphia (695 MW), PSEG/New Jersey (747 MW), BGE/Baltimore (75 MW).

### Overlap Computation

For each destination zone, stress overlap with ComEd is computed as:

Overlap % = (hours where BOTH ComEd and destination are in their respective top-50 for that year) / (ComEd stress hours in that year) × 100

This is computed year-by-year (2022, 2023, 2024, 2025) and as a four-year aggregate. The per-year computation captures temporal variability including extreme event years (2024 Arctic Blast).

### Dynamic Availability

The headline metric: across all 200 ComEd stress hours, how often is at least one cross-region destination zone unstressed?

Dynamic Availability = (stress hours with at least one available destination) / (total stress hours)

## 4. Results

### Control Group: Intra-PJM Proves High Correlation

Before measuring cross-BA decorrelation, we establish the baseline. Intra-PJM zones share common stress drivers: synchronized weather (Midwest-to-East-Coast storm tracks), shared gas infrastructure (Marcellus/Utica pipeline system), and shared AC frequency (Eastern Interconnection).

Approximately half of ComEd’s stress hours find other PJM zones also in their top-50 price hours. If you migrate ComEd workloads to NoVA (DOM), there is a ~36% chance DOM is also stressed. If you migrate to Ohio (AEP), ~45% overlap. Intra-PJM migration helps some of the time, but roughly half the time, the destination is equally expensive. This quantifies the limitation of same-RTO migration and motivates cross-interconnection analysis.

### Headline Result: 99.0% Dynamic Availability

Of the 200 ComEd stress hours across 2022–2025, at least one cross-region destination was unstressed in 198 hours, yielding 99.0% dynamic availability.

The two failure hours cluster in a single event: the January 2024 Arctic Blast, a continental-scale cold event that simultaneously stressed every major grid region. These two hours represent 1% of stress hours and 0.006% of total hours in the dataset.

### Multi-Source Generalizability

The analysis is replicated from three additional source zones to test whether the ComEd-centric result generalizes:

| Source Zone | DC Capacity | Dynamic Availability | Failure Hours |
| --- | --- | --- | --- |
| PJM ComEd | 1,647 MW | 99.0% (198/200) | 2 (Arctic Blast 2024) |
| PJM Dominion (NoVA) | 9,431 MW | 99.5% (199/200) | 1 (Arctic Blast 2024) |
| ERCOT LZ North (DFW) | 4,275 MW | 100.0% (200/200) | 0 |
| CAISO NP15 (Silicon Valley) | 1,311 MW | 100.0% (200/200) | 0 |

Dynamic availability is ≥99% from all four source zones. The single failure mode is the January 2024 Arctic Blast — the only event in four years that produced continental-scale simultaneous grid stress.

### The Interconnection Boundary Effect

The decorrelation structure maps directly onto the electrical topology of the North American transmission system:

Cross-interconnection destinations (ERCOT, CAISO):

ERCOT operates as an electrically isolated interconnection with no synchronous AC ties to the Eastern grid. CAISO sits in the Western Interconnection with temporal offset and weather patterns largely independent from the Midwest.

- ERCOT annual overlap with ComEd: 10–14% (as low as 4–6% in non-Arctic-Blast years)

- CAISO annual overlap with ComEd: 12–14% (zero summer overlap across the full dataset)

- Tail price correlations are negative (ρ = −0.20 to −0.25), meaning destination prices tend to be below average precisely when ComEd prices are above average

The negative tail correlation is the strongest empirical result. It means cross-interconnection destinations are not merely uncorrelated with ComEd stress — they are actively anti-correlated. ERCOT’s stress is driven by Texas-specific weather (summer heat, winter ice) with near-zero correlation to Midwest weather patterns. CAISO’s summer stress comes from September/October fire-weather events, not the July/August Midwest heat dome pattern.

Same-interconnection destinations (MISO, NYISO):

MISO and NYISO operate independently from PJM (separate capacity markets, separate resource adequacy planning), but share the Eastern Interconnection: same synchronized AC grid, shared gas pipeline infrastructure, shared weather exposure.

MISO zone-by-zone overlap with ComEd (four-year average):

| Zone | Overlap (%) | DC Capacity (MW) | Geographic Driver |
| --- | --- | --- | --- |
| MISO Illinois | 66% | 66 | Adjacent to ComEd; shared weather and gas |
| MISO Indiana | 61% | 599 | Adjacent; Great Lakes weather |
| MISO Michigan | 63% | 126 | Great Lakes weather, shared gas |
| MISO Minnesota | 47% | 2,988 | Different weather; still connected gas |
| MISO Arkansas | 43% | 305 | Different weather AND gas |
| MISO Louisiana | 43% | 7 | Different weather AND gas |

The gradient from Illinois (66%) to Louisiana (43%) maps geographic distance from ComEd. The closer the MISO hub, the more correlated its stress. The further south, the more the weather diverges — but even at 43%, same- interconnection zones remain meaningfully correlated.

NYISO zones range from 29% to 41% overlap, intermediate between cross- interconnection (10–14%) and MISO (43–66%), consistent with greater geographic separation from the Midwest but shared Eastern Interconnection membership.

The honest conclusion: The interconnection boundary, not the balancing authority boundary, creates the strongest decorrelation. Cross-interconnect migration is the strongest case; same-interconnect/cross-BA migration provides partial but limited benefit during correlated events.

### Seasonal Decomposition

Stress events have distinct seasonal signatures that affect migration availability:

Summer: - ERCOT overlap with ComEd: 4–6% — essentially uncorrelated - CAISO overlap: 0% — zero coincident summer stress hours across the entire 2022–2025 dataset - MISO: ~52% (same-IC limitation persists regardless of season)

Summer is the cleanest case for spatial migration. Regional heat events (Midwest heat domes) do not typically affect Texas or California simultaneously. Timezone differences may also contribute: summer peaks tend to be short- duration evening events, and the 1–2 hour offset between Central, Mountain, and Pacific time zones means peak periods do not perfectly overlap.

Winter: - ERCOT overlap rises to ~28% in 2024 (Arctic Blast year), reflecting events where continental cold reached Texas. But 72% of ComEd’s winter stress hours still find ERCOT unstressed. - CAISO rises to ~38% — broader winter events affect the West Coast. But 62% of winter stress hours still find CAISO available. - MISO: ~51%, roughly flat across seasons (same IC, same weather, same gas)

Winter is the harder case. Polar vortex intrusions (Elliott 2022, Arctic Blast 2024) can simultaneously affect the Midwest, Texas, and even the West Coast. Gas pipeline constraints propagate across interconnections via fuel- price linkage even where electrical ties do not exist. The two failure hours in the 99.0% dynamic availability result both occur in winter (January 2024).

The seasonal pattern strengthens the thesis for summer migration and identifies winter continental-scale events as the binding constraint. Multi-destination portfolio selection (maintaining readiness across ERCOT, CAISO, and MISO simultaneously) hedges against the winter correlation.

### Year-by-Year Stability

The heatmap (NE paper Fig. 1) reveals that the decorrelation pattern is stable across years, with 2024 as the clear outlier:

- 2022 and 2023: ERCOT overlap 4–6%, CAISO 0%

- 2024: Elevated across the board (ERCOT 26–30%, CAISO 38–44%) driven by the Arctic Blast

- 2025: Returns to baseline (ERCOT 4–10%, CAISO 12–18%)

The 2024 spike is a single-event anomaly, not a regime change. The underlying physical drivers of decorrelation (separate interconnections, independent weather systems, no shared electrical infrastructure) are structural and persistent.

## 5. Pricing Analysis During Stress Hours

### Destination Pricing

During ComEd’s 200 stress hours, destination zone pricing provides indirect evidence on grid conditions at potential migration targets:

- Capacity-weighted mean destination LMP: $165.5/MWh

- Capacity-weighted median destination LMP: $92.49/MWh

- ComEd mean stress-hour LMP: $192.3/MWh

The large mean-median gap reflects a fat-tailed distribution: a few correlated extreme events (Arctic Blast) pull the mean far above the typical experience. An operator with dynamic destination selection — choosing the cheapest available destination in real time — would realistically face closer to the median than the mean.

### Why Energy Arbitrage Is Not the Point

The gross energy spread ($192.3 − $92.5 = $99.8/MWh at the median) is non-trivial but is not the economic driver for spatial migration. The three-prong economic analysis shows that spatial energy-only net value is negative after accounting for destination energy costs during correlated events and operational friction.

The economic case rests on capacity market revenue, not energy arbitrage:

- PJM assigns demand response a 92% ELCC class rating (exogenous to this analysis)

- At the 2027/28 BRA clearing price ($333.44/MW-day), each accredited MW generates approximately $122,000/MW-yr in capacity revenue

- This exceeds any plausible energy spread by an order of magnitude

The stress decorrelation analysis provides the empirical foundation for D1, which determines how deeply the fleet can commit. The capacity market is where that commitment translates to revenue. This is analogous to how a gas peaker earns most of its revenue from capacity payments, not energy sales.

### Low Destination Prices Signal Available Headroom

The negative tail correlation has a secondary implication for the cascade model: low destination prices during source stress signal that destination grids have surplus generation capacity. Surplus generation implies underutilized load, which is consistent with GPU headroom being available at destination data centers precisely when migration demand arrives. This provides indirect support for the D2 parameter (utilization headroom).

## 6. Implications for the Cascade Model

### D1 Parameterization

The stress decorrelation results directly parameterize D1 in two ways:

- Unconditional cascade: D1 = Uniform[0.945, 0.995], central 0.99, derived from the 99.0% empirical dynamic availability with range accounting for climate uncertainty and reflexivity.

- Conditional Monte Carlo: D1 is replaced entirely by observed per-hour, per-destination binary stress flags from the DA LMP data. For each of the 200 ComEd stress hours, the simulation knows exactly which destinations are simultaneously stressed — no parametric assumption needed.

### The Conditional MC’s Additional Information

The conditional MC extracts more information from the pricing data than D1 alone captures. Beyond binary availability, it uses the specific pattern of which destinations are stressed in each hour:

- In some stress hours, all four ERCOT zones plus both CAISO zones are available, providing ~12,000 MW of potential destination capacity

- In others (Arctic Blast), only MISO Louisiana and NYISO Zone A are available, providing <100 MW

- The simulation captures this hour-by-hour variation, which drives the commitment depth distribution’s left tail

This is why the conditional MC produces a distribution of commitment depths rather than a single point estimate, and why the 5th percentile (fleet CVaR) is the relevant metric for resource adequacy planning.

### Why DA LMP Is Methodologically Conservative

Day-ahead LMP is used as a proxy for grid stress throughout this analysis. This choice is methodologically conservative: real-time price spikes are driven by local contingencies (generator forced outages, transmission deratings) that are less correlated across interconnection boundaries than the weather-driven supply-demand imbalances captured by day-ahead prices.

If the analysis were repeated using real-time LMP data, cross-interconnection decorrelation would likely be even stronger, because real-time local contingencies (a single generator tripping in PJM) have essentially zero correlation with real-time conditions in ERCOT or CAISO.

## 7. Limitations

- Historical data only. The 99.0% dynamic availability is an observation from 2022–2025, not a forward guarantee. Climate change may increase the frequency of continental-scale stress events.

- WECC bilateral hubs excluded. Mid-Columbia (~5,554 MW) and Palo Verde (~4,509 MW) are excluded due to daily-resolution pricing. Their inclusion would add ~10,100 MW of strongly decorrelated destination capacity.

- Two MISO zones excluded. Wisconsin and Missouri lack dedicated hub LMPs. Together they represent ~435 MW of operational capacity.

- DA LMP as stress proxy. Price captures supply and demand stress but may miss localized transmission constraints that do not propagate to zonal prices.

- Reflexivity. If spatial migration becomes widespread, migrating load during source stress could increase destination prices and reduce the historical independence. The degree of reflexivity depends on the ratio of migrating load to destination system size — at current data center scales, this effect is negligible, but at the 10+ GW fleet scale modeled in the NE paper, it warrants monitoring.

- Constant load assumption. The analysis treats the new data center load as constant (present in all hours). In practice, data center load has diurnal and weekly variation (documented in the DynamoLLM trace analysis), which would modestly reduce the frequency of stress coincidence.

## 8. Relationship to Other Appendices

This document establishes the empirical basis for D1 (destination availability) and provides the per-hour stress flag data consumed by the conditional Monte Carlo. It connects to the cascade derivation appendix (Pillar 1) through the D1 parameterization and to the policy framework appendix (Pillar 3) through the capacity market revenue calculation.

The key interface points:

- To cascade appendix: D1 central = 0.99, range [0.945, 0.995]

- To conditional MC: per_hour_destination_availability.parquet contract file, containing per-zone MW headroom for each of the 200 ComEd stress hours (zones at zero indicate co-stress)

- To policy appendix: The 99.0% dynamic availability establishes that the physical precondition for spatial migration is met in nearly all observed stress events, motivating the institutional mechanisms proposed in the policy framework

## 9. Evolution from Earlier Versions

This document supersedes archive/Pillar_2.docx. Key changes:

| Change | Old | New |
| --- | --- | --- |
| Destination zones | 20 (including WECC daily hubs) | 17 (hourly-resolution only) |
| MISO structure | 7 LRZ-keyed zones | 6 hub-keyed zones |
| MISO Illinois capacity | 1,647 MW (ComEd’s value, double-counted) | 66 MW (Ameren IL only) |
| Iowa pricing | Michigan Hub | Minnesota Hub |
| Dynamic availability | 94.5% (189/200) | 99.0% (198/200) |
| NYISO | Not included | 5 zones, 891 MW |
| MISO overlap range | 43–63% | 43–66% |
| Failure hours | 11 (Arctic Blast cluster) | 2 (Arctic Blast cluster) |

The most consequential change is the improvement in dynamic availability from 94.5% to 99.0%, driven primarily by the addition of NYISO zones (which provide additional Eastern Interconnection destinations with 29–41% overlap) and the correction of the MISO zone metadata (which removed the MISO_ILLINOIS double-count and fixed Iowa’s hub assignment). The reduction in failure hours from 11 to 2 reflects that the additional NYISO destinations were available during 9 of the 11 previously all-stressed hours.

## References

Baxtel. US Data Center Database. baxtel.com (accessed 2025).

CAISO. OASIS Day-Ahead Market Data. oasis.caiso.com.

ERCOT. Day-Ahead Market Settlement Point Prices. ercot.com.

MISO. Day-Ahead Market Hub LMP Data. misoenergy.org.

NERC. 2024–2025 Winter Reliability Assessment (2024).

NYISO. Day-Ahead Market LBMP Data. nyiso.com.

PJM Interconnection. Data Miner: Day-Ahead Locational Marginal Pricing. dataminer2.pjm.com.

U.S. Department of Energy / National Renewable Energy Laboratory. Accelerating Speed to Power Data Viewer. maps.nrel.gov/speed-to-power (accessed December 2025).